Sierra County

Water Shortage Emergency Response Process

Draft

November 2025

# Introduction

This Water Shortage Emergency Response Process (Water Shortage ERP) documents how Sierra County will monitor water supply conditions to set an appropriate water shortage stage with corresponding short-term response actions to mitigate the impacts or potential impacts of a water shortage to domestic wells and state small water systems (SSWS).

# Water Shortage Stages—Indicators and Triggers

The Water Shortage ERP is organized into three water shortage stages: Information, Alert, and Response. The water shortage stage is informed using Table 1, which lists the indicators (water supply-related features) and their associated triggers (conditions indicating a water supply status) to determine the current water shortage. These indicators and triggers can be used to support a water shortage stage recommendation and actions to the Task Force and public, as well as an emergency declaration for State and Federal resources. The information in this table is not exhaustive and is intended to show how the indicators and triggers could be considered. Other indicators and triggers (not shown) may also be developed by the County. The County will evaluate its triggers in April and make a corresponding water shortage stage recommendation to the Task Force at its annual meeting held in May. However, if conditions change considerably during the winter or at any time, the County can reassess as necessary.

# Responding to a Water Shortage Stage

The County has aligned specific activities to each water shortage stage, as shown in Table 2. New activities not listed in Table 2 may be identified and implemented based on actual shortage conditions. In addition, not all activities listed may be implemented during a shortage event. The County should consider the nature of the shortage when determining what activities are most appropriate.

In the “Information Stage,” the County would complete an annual review of the County DRP Implementation Plan, evaluate the water supply system health, organize a meeting with the Task Force, update County resources (i.e. website, vendor lists, etc.), and continue proactive outreach to domestic well and SSWS. In the “Alert Stage,” the County would focus on raising awareness of potential water shortage issues, providing the steps to follow if a water shortage occurs, encouraging temporary mitigation measures that may avoid a shortage, and coordinating with other organizations that may be involved in the event a shortage occurs (such as the Sierra Valley Groundwater Management District, public water systems, and other County departments). In the “Response Stage,” the County would evaluate if emergency water supplies are necessary, coordinate with other organizations, and update the Task Force.

# Responsibility and Coordination

Table 2 lists the responsible entity for activities that could be implemented in each water shortage stage and indicates other entities that may be involved or with which coordination would be needed.

Table 1. Indicators and Trigger Considerations for Determining Active Water Shortage Stages for Domestic Wells and SSWS in Sierra County

| Indicator Name | Indicator Description | Trigger Consideration When Going into the Alert Stage | Trigger Consideration When Going into the Response Stage |
| --- | --- | --- | --- |
| US Drought Monitor | The US Drought Monitor classifies drought conditions across the United States as Abnormally Dry, Moderate, Severe, Extreme, and Exceptional, and is updated every Thursday. This classification can be used to evaluate regional drought conditions near the County. | The County would enter the Alert Stage if the US Drought Monitor reaches Moderate status in neighboring counties or within the County. | If the County is classified as being in an Extreme or Exceptional drought and there is additional indication of imminent water shortage, the County would enter the Response Stage. |
| Public Water System Water Shortage Stage | A Public Water System is required to maintain a Water Shortage Contingency Plan (or have one within an Urban Water Management Plan) that specifies stages of water shortage tied to projected water supply shortfalls. The Drought Stages that Public Water Systems may be a proxy for overall drought conditions within the County. | If a Public Water System is in a Drought Stage, the County may consider entering the Alert State to ensure consistent messaging and reduce water use. | A Water Shortage Contingency Plan must include an Emergency Stage to be used only when water supply shortages are imminent. If a Public Water System in the County is in an Emergency Stage, the County would consider entering an emergency stage based on similarities in water supplies. |
| Water Quality | Degradation of water quality can lead to water shortage if water becomes unsuitable for human consumption, particularly if no backup supply is available. Water quality may also be impacted by external hazards such as heavy rainfall, wildfires, and droughts. | Water quality-related water shortages have historically occurred in spring fed systems after major rainfall events. After major rainfall events and when systems are in violation of drinking water standards, the County will enter the Alert Stage. | The County monitors water quality conditions in at-risk spring fed systems. When those systems are in violation of drinking water standards for three or more days, the County will enter the Response Stage. |
| Non-Drought Hazards | Other hazards such as wildfires, earthquakes, floods, and power outages (either planned or unplanned) can lead to water shortages and impact water quality, both of which can be amplified by drought conditions. | Hazards that temporarily interrupt domestic well or SSWS supplies for up to two days will trigger the Warning Stage. | Hazards that interrupt domestic well or SSWS supplies for an extended period (3 or more days) will trigger the Emergency Stage. |
| Applications for New Well Permits | Increases in well permit applications for existing well replacement or rehabilitation can indicate low water levels or water shortages. | When the County is experiencing a drought and there is an increase in well permit applications for existing well replacement or rehabilitation in a concentrated area, the County will enter Alert Stage and monitor water availability in the affected regions. | If the County finds widespread water shortages in regions with an increase in well permit applications for existing well replacement or rehabilitation, the County will enter the Response Stage. |
| Dry Well Reports | California has a dry well reporting system that well owners or operators can use to report a dry well. These reports can inform where low water levels and/or water shortage may be present. | The County will monitor potential causes of reported dry wells and will enter the Alert Stage if a dry well is found to have been exacerbated by drought conditions. | If features of the dry well(s) (i.e., depth, age, etc.) are consistent with other wells in the area, low water levels and/or water shortage may impact a wider area than reported. The County will consider similarity and judgment to determine when to enter the Response Stage. |
| GSA Monitoring | The Sierra Valley Groundwater Management District monitors groundwater levels in the Sierra Valley Groundwater Subbasin during April and October as part of its GSA reporting requirements. This data can be used to track potential water shortage within that groundwater basin. | The County will enter the Alert Stage if monitoring or analysis by the Sierra Valley Groundwater Management District indicates a decline in groundwater levels below the established thresholds as defined in the GSP. | If the Sierra Valley Groundwater Management District implements response actions to avoid water shortages, the County will consider entering the Response Stage. |

Key:
County = Sierra County
GSA = Groundwater Sustainability Agency
GSP = Groundwater Sustainability Plan
SSWS = State Small Water Systems

Table 2. Water Shortage Emergency Response Process Activities Aligned to Water Shortage Stage with Responsible Organization(s) for Sierra County

| Water Shortage Stage | Description | Activity Description | Responsible Organization | Comment |
| --- | --- | --- | --- | --- |
| Information | No major drought or potential for water shortage | Annual County DRP Implementation Check In | OES | OES will review the status of the DRP Implementation Plan, determine if any modifications are required, and assign responsibilities for making DRP modifications. Review and update contacts as needed. |
|  |  | Annual Water Supply Assessment | OES | OES, in partnership with others, will evaluate the water supply system health in April to support a water shortage stage recommendation for the Task Force. |
|  |  | Annual Task Force meeting | OES | Annual Task Force meetings will occur in May after the water supply health is evaluated. |
|  |  | Annual website update | OES | Website will be updated in November and April to reflect current conditions. |
| Alert | Drought or other hazard occurring that could cause a water shortage | As-Needed Task Force Meetings | OES | OES will schedule additional Task Force meetings based on hazard. |
|  |  | Outreach to domestic well and SSWS communities on water shortage-related information, including, but not limited to: - How to monitor domestic well health - Resources for when a well runs dry - County contact information - Information on water shortage conditions within the County | OES | OES will outreach to domestic well and SSWS communities within the County with water shortage-related information. This outreach could be via website updates, social media posts, email distributions, and other methods of communication. |
|  |  | As-needed coordination meetings with relevant organizations such as: other County departments, Sierra Valley Groundwater Management District, local water providers, power companies, and other agencies | OES | OES will initiate coordination and schedule as-needed check-in meetings with relevant organizations to discuss and monitor water shortage conditions and potential activities. |
| Response | Water shortage is occurring or is imminent | Regular Task Force meetings | OES | OES will schedule additional Task Force meetings. |
|  |  | Activate Emergency and Interim Drinking Water Distribution Plan as listed in this County DRP for area(s) experiencing shortage | OES | OES will activate the Emergency and Interim Drinking Water Distribution Plan, but other entities will implement specific activities as defined. |
|  |  | Standing coordination meetings with relevant organizations | OES | OES will organize regular coordination meetings and will delegate to other organizations as required. |

Key:
County = Sierra County
DRP = Drought Resilience Plan
OES = Sierra County Office of Emergency Services
SSWS = State Small Water Systems
Task Force = Sierra County Drought and Water Shortage Task Force